Firmware Upgrades for Clans Medpro’s and Val Boards

Login to the Definity 
Determine the IP address of the source circuit pack. You can either ask the data systems administrator or enter the following SAT command sequence:
a.Type display ip-interfaces. Find the source circuit pack’s Slot location and write      down its Node Name.
b.Type display node-names ip. Find the source circuit pack’s Node Name and write down its IP Address .
Set up source circuit pack’s file system
Before files can be copied to the source circuit pack, you must set up a file system on the source circuit pack to accept the files.
1.Log into the SAT using a superuser or services login and password.
2.Enter enable filesystem board UUCSS login ftplogin ftppassword, where:
■UUCSS is the location of the source circuit pack.
■ftplogin is any temporary login ID that you choose for this FTP session (for example, denver).
■ftppassword is any password that you choose for this FTP session (for example, broncos). The password must be at least 7 characters long.
Later, you will use this same ftplogin and ftppassword in the following section, ‘‘FTP the image file to source’’. With the WSFTP Exploror
This completes the preparation for a firmware download in the Definity Application.

Open up the WS_FTP Pro Exploror


Go to My sites


Set up new site


Name the site you are going to use


Put the IP Address of the Clan you are going to use from the Definity prep you just did

Enter the login name you entered above… (example of denver)


Enter the Password you entered above…  (example of broncos)

Click on Save Password


And finish


Now we can open the site we just created by double clicking


Once we here the whoo/whoo sound and the folder is open we can go open the location

 we have stored the file we are going to send up to the Clan for use to upgrade

We drag the file we need into the new open window with the name we named it

Once it says it is done and we see the file in the open window we have completed

Using the WS_FTP Exploror
Now back to the ASA Screen

Enter change firmware download and fill in the fields as described in Table 3.
If you are performing a self download, you will be entering only one target board location and it will be the same as the source board location. Screen 2 provides an example of self download.
If you are performing a CLAN-distributed download, you will be entering a C-LAN location as the source and 1 - 50 target locations, where the target boards are all of the same type. Screen 3 provides an example of a C-LAN-distributed download.
[image: image1.png]/ehange firmvare download Page 1 of
sourcs soard Location: 0104
Pirmare Inage 7ile Name: uedivzrl
Target Bosrd Goda; TNAGL SUFfix: GP Firmesre vintags.
sehedule Downloads y Remove Inage Pile After Successful Dowmloads y
Start Date/Tive: 10/08/2002 22,00 Stop Date/Tine: 10/08/2002 22:30

Targer Targer Target Target Targst
Location Location Location Lozation Lozarion
T 01n04 frons pron Frowan e
20 01m0s ri— Frie e e —

Enter 5 character board mumber; cabinet (1-30) scarrier (3-B) :slot (0-20)





If it fails we need to do a Test firm down to clear the failure. And try again

